# CBD Isolation by Crystallization

# I. Purpose

The following procedures describe the isolation of chemically pure CBD by crystallization using pentane.

# II. Objective

The objective of these procedures is to prioritize the safety of personnel while producing a consistent, quality controlled final product.

# III. Responsibility

This SOP is the responsibility of the following personnel:

- Director of Extraction

- Extraction Technician

# V. Definition of Terms

The following terms are used in this document:

Personal Protective Equipment (PPE) - Specialized clothing or equipment worn by employees for protection against health and safety hazards.

Standard Operating Procedures (SOP) - established or prescribed methods to be followed routinely for the performance of designated operations or in designated situations —called also standing operating procedure.

CBD Isolate – Purified CBD in crystalline form.

Mother Liquor – The liquid mixture remaining after crystal filtration.

Pentane – A non-polar solvent with a flash temperature of 57F/14C boiling temperature of 97F/36C

Distilled Oil – In this application Distilled Oil will mean winterized, decarboxylated and distilled oleoresin of the cannabis plant containing 70% or more CBD by weight.

## Materials Needed:

- Stainless Steel Mixing Vessel

- Stainless Steel overhead or hand mixer/homogenizer

- Mortar & Pestle or stainless steel grinder (manual, pneumatic, or appropriately rated electric motor)

- Stainless Steel mesh screen basket (as large as opening of stainless steel vessel)

- Dry Ice/Alcohol Slurry & Storage Bucket (capable of accommodating stainless steel mixing vessel)

- Chemically Pure Pentane

- Distilled Oil containing 70% or more CBD by weight

## Personal Protective Equipment:

  - Gloves

  - Lab coat

  - Safety glasses

  - Shoe covers

  - Hairnet or lab hat

  - Beard cover

Note: All PPE must be worn for the duration of all procedures.

Note: Any employee who will be conducting work in the Laboratory must be trained on “SOP - Chemical Hygiene Plan”, before any process can be started. Also training must be completed on “SOP – Pentane Use” before the solvent can be used in any process.

# VI. Procedure

## Preparing Distilled Oil

  - Fill a clean stainless steel vessel no more than half way with Distilled Oil

  - Warm the Distilled Oil in the vessel to 90F/32C, stirring constantly with overhead mixer or homogenizer on lowest setting.

- Mixing Pentane

  - Slowly introduce room temperature pentane while constantly stirring the distilled oil on the lowest mixer/homogenizer setting.

    - Do not allow the mixture to exceed 90F/32C

    - Do not mix pentane outside of an appropriately ventilated environment.

    - Do not mix pentane without wearing all required PPE.

  - Continue to mix until the distilled oil is uniformly distributed within the pentane.

- Chilling the Solution

  - Using an appropriately rated, solvent safe freezer or dry ice slurry, slowly bring the solution down to -31F/-35C

    - It is important to decrease solution temperature slowly to achieve the cleanest possible initial crystal formation.

  - Allow solution to remain at -31F/-35C until substantial formation of crystals is visible at the bottom of the stainless steel vessel.

- Removing Mother Liquor

  - Pour the chilled solution out of the stainless steel mixing vessel into another stainless steel mixing vessel through a fine stainless steel mesh screen.

  - Recover the pentane from the mother liquor using an appropriate solvent recovery system and set aside the remaining oil to be refined later.

- Rinsing the Crystals

  - Chill pentane to -31F/-35C in appropriately rated freezer or in dry ice slurry.

  - Remove CBD Crystals from the bottom of the stainless steel mixing vessel and place in a stainless steel mesh screen basket resting securely over the mother liquor solution.

  - Pour the chilled pentane over the crystals, agitating/stirring as little as necessary to rinse all crystals until they are completely colorless.

    - Use as little pentane as necessary to remove impurities.

    - Use a wash bottle with a nozzle to maximize efficiency.

    - Ensure the pentane stays chilled to -31F/-35C during the entire rinsing process.

- Pulverize the Crystals

  - Chill the CBD Crystals in an appropriate freezer or dry ice slurry to -31F/-35C

  - Remove from freezer/slurry and filter any residual liquid pentane through a stainless steel mesh screen.

  - Return crystals to stainless steel vessel and pulverize into a uniform powder using a mortar and pestle, or a manual, pneumatic or appropriately rated electric grinder.

  - Evenly distribute the CBD powder over a large flat surface area (in stainless steel trays lined with parchment or shallow pyrex dishes) at a thickness of approximately 1inch/2.5cm

  - Place the CBD powder in a vacuum oven at ambient/room temperature and pull vacuum in one or two hour intervals until the residual pentane is removed.

    - Break vacuum only to stir and redistribute the powder as needed.

# Change Log

| Date | Revision | Modified By | Description of Changes |
| --- | --- | --- | --- |
| [Date] | 1.0 | [Department F/-Head] | Initial release. |

Document Release

| Name: | Signature: | Date: |
| --- | --- | --- |